1.1 COVERING LETTER

Date: 	05/05/2026

DRN:	VKG2025-26/FA0XX

To,

The Compliance Department,

Trent Ltd – A TATA Enterprise,

28th Floor, LODHA Excelus, New Cuffe Parade (NCP),

Sewri-Chembur Road, Near IMAX Dome,

Eastern Freeway, Wadala, Mumbai – 400037.

SUBJECT- FIRE AUDIT REPORT FOR AUDIT CARRIED OF FIRE AND LIFE SAFETY MEASURES IMPLEMENTED

IN PREMISES FOR ASSET PROTECTION, PUBLIC LIFE SAFETY AND ENVIRONMENTAL HAZARD PREVENTION.

We M/s. VKG Fire and Safety are thankful to Trent Ltd - A TATA Enterprise for their Co-Operation and help without which this Fire & Safety Audit could not have been possible. The Courtesy and Cordiality Extended to us is Highly Appreciated. In the above said Fire and Life Safety Audit we have covered the below mentioned Inspections and Audits of the Store as committed:-

- Basic Fire Safety Observation and Inspection of Complete Premises

- Fire Fighting Provisions Executed in Store and Entire Property

- Fire Safety Audit and Report Drafting as per NBC Part IV of Fire & Life Safety 2016

- Inspection and Operation of Installed Fire Fighting Equipments

- Hazard Identification and Risk Assessment

- Conclusion Report of Executed Fire Audit

We are submitting herewith the necessary documents of Fire Safety Audit as per the applicable Fire Safety Norms and Statutory Requirements.

Regards,

M/s. VKG Fire and Safety

1.2 ACKNOWLEDGEMENT

We, M/s VKG Fire and Safety would like to express our sincere heartfelt gratitude to M/s. Trent Limited for awarding us the work of Fire Safety Audit alongwith M/s. Trent Limited we are thankful to below delegates and firms for consulting/assisting us in carrying the Store Survey, Store visit, Fire Inspection, Height Verification of Property, Electrical Inspection, Fire Audit, Risk Assessment, Hazard Identification, Checking of Installed Fire Fighting Equipments, Report Drafting and consultation pertaining to FSI & NBC Guidelines.

M/s. Kayansh Fire & Safety Solutions

M/s. Greentech Engineers Services

M/s. Hindustan Fire

M/s. Genesecure Technologies

M/s. Redair Fire Technologies

M/s. GreenFeather Safety & Security Solution

Mr. Chinmay Bhosale – Certified Architect

Mr. Nilesh Waghmare - Certified Electrical Auditor

Key People Involved in Fire Audit:

Height Inspection Verified by – 		Mr.Saroj Debendranath Rauta

Height Certificate Issued By-			Mr. Chinmay Bhosale – Certified Architect

Fire Audit Carried by -				M/s.VKG Fire & Safety (and associates)

Audit Report Designed by-			Mr.Dheeraj Ramand Pardeshi

Observations & Recommendations -		Mr.K.P. Jadhav (Retd. Station Fire Officer)

Fire Audit Report Issued by -			Mr.Shivam Deoghare (FSAI – Accredited Professional)

1.3. INDEX

| Sr No. | Reference | Description | Page |
| --- | --- | --- | --- |
| 1.) | 1.1 | COVERING LETTER | 01 |
| 2.) | 1.2 | ACKNOWLEDGEMENT | 02 |
| 3.) | 1.3 | INDEX | 03 |
| 4.) | 1.4 | SUMMARY | 04 |
| 5.) | 1.5 | OVERVIEW | 05 |
| 6.) | 1.6 | OBJECTIVE & METHODOLOGY | 06 |
| 7.) | 2.1 | NBC INTRODUCTION AND OVERVIEW | 07 |
| 8.) | 2.2 | NBC GUIDELINES FOR F1 & F2 TYPE OCCUPANCIES | 09 |
| 9.) | 2.3 | NBC TABLE 7 SUMMARY | 11 |
| 10.) | 3.1 | PROPERTY OVERVIEW | 12 |
| 11.) | 3.2 | BUILDING DETAILS | 13 |
| 12.) | 3.3 | FIRE EQUIPMENTS CHECKLIST - BUILDING | 14 |
| 13.) | 3.4 | STORE DETAILS | 15 |
| 14.) | 3.5 | FIRE EQUIPMENTS CHECKLIST - STORE | 16 |
| 15.) | 4.1 | FIRE FIGHTING EQUIPMENTS DETAILS | 17 |
| 16.) | 5.1 | STORE INSPECTION SUMMARY | 23 |
| 17.) | 5.2 | HAZARD IDENTIFICATION | 24 |
| 18.) | 5.3 | OCCUPANCY LOAD DERIVATION | 25 |
| 19.) | 5.4 | FIRE LOAD CALCULATION | 26 |
| 20.) | 6.1 | NBC NECESSITY | 28 |
| 21.) | 7.1 | EMERGENCY EVACUATION CHECKLIST | 35 |
| 22.) | 7.2 | SMOKE VENTILATION CHECKLIST | 36 |
| 23.) | 7.3 | ELECTRICAL SAFETY CHECKLIST | 37 |
| 24.) | 7.4 | MAINTENACE TRAINING CHECKLIST | 38 |
| 25.) | 8.1 | RISK IDENTIFICATION | 40 |
| 26.) | 8.2 | OBSERVATION AND REMARKS | 41 |
| 27.) | 8.3 | AREAS OF IMPROVEMENT | 42 |
| 28.) | 9.1 | CONCLUSION | 43 |
| 29.) | 9.2 | LOCATION WISE COMPLETE EQUIPMENT REPORT | 44 |
| 30.) | 9.3 | IMAGES FOR REFERENCE |  |

1.4 SUMMARY

1.4.A)

This is to certify we have carried Fire Detection, Protection, Active and Passive Firefighting System Equipments Survey, Fire Fighting Maintenance, Fire Equipment Testing and Fire Audit work of your premises. Work was carried from the period of 25th April and completed on 04th May 2026.

1.4.B)

We have currently completed Fire Audit work and therefore handing over you the report of Fire Fighting System Installed in your premises and necessary actions to be taken by Store/Property to keep the premises Safe and Secure from Fire & Life Safety Point of view.

1.4.C)

Fire Audit of the premises is carried in phases, Several SOP’s & Protocols followed while carrying the Audit have been discussed further in report. Our objective of carrying the Fire Audit to understand the risk factors involved in premises pertaining to Fire & Life Safety point of view.

1.4.D)

Any observations, remarks, statement made in the report are completely professional and made from safety point of view under the provisions and rules set by National Building Code of India – Part 4: Fire and Life Safety (Bureau of Indian Standards).

1.4.E)

Before going thru the report we would like to highlight, we do not intend to determine any individual’s mistake or remark anyone personally in the report and strongly condemn such act which may lead to contemptibility of individual’s identity, reputation or social status and therefore recommend to keep the report highly confidential and discuss on the solutions accordingly.

1.4.F)

This report is created on observing the firefighting system installed in premises on fair ground and do’s, don’t pros and cons statement mentioned in report are strongly affiliated from guidelines of National Building Code of Part IV of Fire and Life Safety Laws of 2016.

1.5 OVERVIEW

1.5.A)

A fire safety audit in mercantile premises is a comprehensive assessment of the fire safety measures and procedures in place to protect residents and property from fire hazards. It is essential to ensure the safety of residents and compliance with local building and fire safety codes.

1.5.B)

In Fire Audit we determine the scope of potential fire hazard that may occur in premises including the number and types of residential units, common areas and other areas that need to be inspected for Public Safety, Prevention of Life and Loss of Property & Assets.

1.5.C)

While carrying Fire Audit of premises we inspect fire protection equipment such as fire extinguishers, smoke detectors, fire alarms, sprinkler systems, hydrant system and other passive fire protection equipments and ensure they are in good working condition and meet regulatory requirements.

1.5.D)

In Fire Audit we Assess Building Layout, Egress Time and Evaluate the layout of the premises, including escape routes, exits, stairwells, and hallways to ensure they are clear, well-lit& free from obstructions in case of emergency or any fire hazard.

1.5.E)

We carry examination of electrical systems and heating equipment to identify potential fire hazards. Look for overloaded circuits, damaged wiring, and faulty appliances, identify and address common fire hazards, such as flammable materials, blocked fire exits, and improper storage of combustible materials.

1.6 OBJECTIVE & METHODOLOGY

1.6.1) The objectives of Fire and Safety Audit are as Follows:-

1.6.1.a) Examine the existing fire and safety measures, procedures, system for controlling measures.

1.6.1.b) Identify potential hazards which have caused or are likely to cause personal injury

1.6.1.c) Recommend on the basis of identified hazards, changes to improve upon the existing system

1.6.2) The following methodology has been adopted to Achieve the Objectives:-

1.6.2.a) Practically visiting every Floor in premises.

1.6.2.b) Visiting every nook and corner of the Premises.

1.6.2.c) Collecting data of all the existing Firefighting Equipments

1.6.2.d) Preparing report based on actual finding

PLEASE NOTE:

BELOW PROVIDED REPORT IS BASED ON OUR FINDINGS AND THE NECESSARY ACTIONS RECOMMENDED IN THE REPORT ARE EXTRACTED FROM THE RULES AND REGULATIONS ABIDED BY LOCAL FIRE DEPARTMENT BASED ON BUILDING DESIGN CODES MENTIONED IN NATIONAL BUILDING CODE OF PART 4 OF FIRE AND LIFE SAFETY. DECISION TO EXECUTE THE SAFETY MEASUREMENT ACTIVITIES IS CHOICE OF OWNER/ OCCUPIER/ RESIDENT. HOWEVER ANYBODE REFUSE TO FOLLOW NECESSARY PRECAUTIONARY STEPS WILL BE HIMSELF/HERSELF LIABLE FOR THE LOSS OF PROERTY/LIFE IN CASE OF FIRE HAZARD AND LICENSED AGENCY/ AUDITOR WILL NOT BEAR ANY CONSEQUENCES OR ACTIONS.

2.1.NBC INTRODUCTION AND OVERVIEW

NBC Part 4 – Fire and Life Safety is a section of the National Building Code of India (NBC) 2016, which outlines the minimum fire prevention, life safety, and firefighting requirements for all buildings and structures in India.

### Objective:

To ensure minimum safeguards against fire and related hazards in buildings to:

- Protect human life

- Minimize property damage

- Ensure safe evacuation

- Facilitate emergency response

## Scope:

NBC Part 4 applies to:

- All types of buildings, including residential, commercial, industrial, institutional, mercantile, assembly, storage, hazardous, etc.

- Covers design, construction, occupancy, and maintenance of buildings with respect to fire safety.

## Key Sections of NBC Part 4:

### 1. Classification of Buildings (Occupancy-Wise)

- Group A: Residential

- Group B: Educational

- Group C: Institutional

- Group D: Assembly

- Group E: Business

- Group F: Mercantile (includes clothes stores)

- Group G: Industrial

- Group H: Storage

- Group J: Hazardous

### 2. Life Safety Provisions

Focuses on safe evacuation of occupants:

- Exit requirements (number, size, travel distance, width)

- Staircases and corridors

- Emergency lighting

- Exit signage

- Smoke control and compartmentation

### 3. Fire Prevention

- Construction materials must be fire-resistant (as per rating: 1-hour, 2-hour, etc.)

- Electrical systems, kitchens, storage areas must be properly designed and monitored.

### 4. Fire Protection Installations

Specifies fire protection systems by building use and height:

- Portable fire extinguishers

- Automatic sprinkler systems

- Fire detection and alarm systems

- Wet riser/hydrants

- Smoke detectors

- Fire lifts

- Fire command center

- Water storage for firefighting

## Importance of NBC

While NBC is a model code adopted by government fire authorities and other Indian Standard bodies as NBC lays down minimum safety standards for fire, structural stability, sanitation, lighting, and ventilation. NBC helps to prevent building collapses, fires, and other life-threatening hazards. NBC Part 4 ensures fire safety and safe evacuation during emergencies.

2.2.NBC GUIDELINES TO FOLLOW FOR MERCANTILE STORES

National Building Code of India (NBC), 2016 – Part 4: Fire & Life Safety specifies for a merchandise/retail store such as a clothes store, which falls under Group F: Mercantile Buildings (subdivision F-1 or F-2)

Merchandise/Retail Stores (including clothes stores) are categorized under Group F (Mercantile):

(NBC Ref Page 11, Point 3.1.7)

F-1: shops, stores, departmental stores up to 500 m².

F-2: shops, stores, departmental stores over 500 m².

2.2.1 Life Safety Requirements for Group F

- Exits:

At least two separate exits are mandatory for every floor, including basements. These exits must be remote from each other, accessible via different travel paths, with a common path permitted only for up to 15 meters from any point.

- Hazardous Areas:

Any hazardous sections within the store (like storage areas with flammable materials) must be segregated or suitably protected.

- Open Aisles / Self-service:

In self-service layouts, checkout counters or barriers must not obstruct exits or required aisles.

2.2.2 Fire Active/Passive Protection Requirements for Group F

- Automatic Fire Detection & Alarm:

Buildings of Group F occupancy should have an automatic fire detection and alarm system installed.

- Fire Exit and Fire Door

Fire Exit Door shall be provided for area beyond 15 m2.

2.2.3 Fire Fighting Equipment Requirements for Group F

Provision of

- Fire Extinguishers,

- Fixed Fire Hydrants,

- Sprinklers or Fire Suppression Systems

is necessary as mentioned in Table 7 of Part 4.

2.2.4 Additional Safety Measures (General NBC Part 4 Highlights)

- Escape Lighting & Exit Signage:

Exit routes must be clearly marked with illuminated signage.

Emergency lighting must activate within 5 seconds of normal lighting failure and last at least 90 minutes, delivering a minimum of 10 lux at floor level, reducing to 5 lux over half the width for routes up to 2 m wide.

- Fire Drills:

Conduct fire drills every six months, although during the first two years of occupancy they may be conducted every three months.

- Smoke Control & Ventilation:

Air handling systems must prevent smoke spread if serving multiple floors or compartments, triggered by a fire alarm.

- Compartmentation, Fire-rated Openings:

Vertical openings (e.g., stairs, ducts) must be properly enclosed with fire-resistant assemblies (typically 2-hour rating). Openings in fire-resistant walls must be protected by fire doors rated for 2 hours; openings greater than 5.6 m² or 2.75 m in height/width have special requirements.

2.2.5. Other mandatory Requirement for Mercantile Stores:

- Install proper exit systems:

Make sure there are two well-separated exits and pathways under any layout or layout changes.

- Address special areas:

If you have storage or other potentially hazardous zones, ensure they're separated with fire-rated barriers or protections.

- Optimize store layout:

Especially in self-service arrangements, design checkout and display areas so they don’t block escape routes.

2.3.NBC TABLE 7 SUMMARRY

| Category | Requirement |
| --- | --- |
| Occupancy Classification | Group F (Mercantile) – clothes store |
| Number of Exits | Minimum 2 separate, remote exits; ≤15 m common access allowed |
| Self-service Layouts | No obstruction to exits/aisles by counters |
| Hazardous Areas | Must be segregated or protected |
| Fire Detection & Alarm | Automatic systems mandatory |
| Firefighting Equipment | Extinguishers + hydrants/sprinklers per Table 7 |
| Escape Lighting & Signage | Illuminated signs, emergency lighting with ≥10 lux |
| Fire Drills | Every 6 months (3 months initially) |
| Smoke/Ventilation Control | Prevent spreading via ducts/air systems |
| Compartmentation | Fire-rated enclosures and assemblies for verticals/openings |

Minimum requirement of Fire Fighting System as per table 7 for F1 & F2 Type Occupancies

(Table 7 Summary)

| Store Type / Height Classification | Fire Alarm System (MOEFA) | Fire Extinguisher | Down Comer Fire Hydrant | First Aid Hose Reel | Fire Pump |
| --- | --- | --- | --- | --- | --- |
| Ground + 1 floor, ≤ 500 m² – F-1 | Required | Required | Required | Required | 450 LPM |
| Ground + 1 floor, > 500 m² – F-2 | Required | Required | Required | Required | 900 LPM |
| More than Ground + 1 floor – F-2 | Required | Required | Required | Required | 900 LPM |
| Above 15 m but ≤ 24 m | Required | Required | Required | Required | Required |
| Above 24 m but ≤ 30 m | Required | Required | Required | Required | Required |

3.1.PROPERTY OVERVIEW

1. Property Information

| Field | Description |
| --- | --- |
| Property Name: |  |
| Address: |  |
| Owner/Occupier Name: |  |
| Contact Details: |  |
| OC Number: |  |
| Registration Jurisdiction: |  |
| Type of Property: |  |
| Total Area of Property: |  |
| Number of Floors: |  |
| Marginal Space in Front: |  |
| Marginal Space in Left: |  |
| Marginal Space in Right: |  |
| Marginal Space Behind: |  |
| Stilt Parking Details: |  |
| Basement Details: |  |

3.2.BUILDING DETAILS

2. Building Design, Details, Utilities, Services, Features and Overview

| Field | Description |
| --- | --- |
| Type of Construction |  |
| Year of Construction |  |
| Height of Building (m) (Ground to Terrace) |  |
| Presence of Refuge Area (s) |  |
| Type of Staircases |  |
| Openings and Ventilation |  |
| Compartmentation |  |
| Elevator Details: |  |
| Fire Exit Details: |  |
| Electrical Supply Details: |  |
| Gas/Flammable Substance: |  |
| HVAC/AC Systems: |  |
| Water Supply |  |
| Safety & Security Surveillance: |  |
| Automated Doors & Exits: |  |

3.3.BUILDING FIRE EQUIPMENT CHECKLIST

| Field | Description |
| --- | --- |
| Fire NOC Details |  |
| Fire Pump Details |  |
| Fire Hydrant System Details |  |
| Fire Sprinkler System Details |  |
| Fire Extinguisher Details |  |
| Fire Alarm System Details |  |
| Public Address System Details |  |
| Fire Safety Signage: |  |
| Fire Door: |  |
| Dedicated Evacuation Point: |  |
| Emergency Exit: |  |
| Fire Staircase: |  |
| Emergency Elevator: |  |
| Fire Refuge Area: |  |
| Fire Escape Plan: |  |

3.4.STORE DETAILS

3.4. PRIMARY STORE DETAILS

| Field | Description |
| --- | --- |
| Name of Store: |  |
| Store Address: |  |
| Nature of Business: |  |
| Type of Use: |  |
| Nature of Goods/Processes: |  |
| Total Built-up Area (m²) |  |
| Number of Floors |  |
| Floor-wise Area |  |
| Occupancy Classification (NBC) |  |
| Business Registration Details: |  |
| Year of Establishment: |  |
| Store Contact Details: |  |
| Total No of Employees: |  |
| Hours of Operations: |  |
| No of Security Personnel: |  |

3.5.STORE FIRE EQUIPMENTS CHECKLIST

| Field | Description |
| --- | --- |
| Fire NOC Details |  |
| Fire Pump Details |  |
| Fire Hydrant System Details |  |
| Fire Sprinkler System Details |  |
| Fire Extinguisher Details |  |
| Fire Alarm System Details |  |
| Public Address System Details |  |
| Fire Safety Signage: |  |
| Fire Door: |  |
| Dedicated Evacuation Point: |  |
| Emergency Exit: |  |
| Fire Staircase: |  |
| Emergency Elevator: |  |
| Fire Refuge Area: |  |
| Fire Escape Plan: |  |

4.1.FIREFIGHTING EQUIPMENTS DETAILS

| 4.1.A | WATER TANK DETAILS:- | Details | Additional Remarks |
| --- | --- | --- | --- |
| a.) | Building Common Water Tank Details: | YES/NO |  |
| 1.) | U/G Water Tank Capacity of Building? |  |  |
| 2.) | O/H Water Tank Capacity of Building? |  |  |
| b.) | Building Fire Tank Details: | YES/NO |  |
| 1.) | Does Building have separate fire water tank? |  |  |
| 2.) | If Yes, U/G Water Tank Capacity of Building? |  |  |
| 3.) | If Yes, O/H Water Tank Capacity of Building? |  |  |
| c.) | Store Water Storage Details: | YES/NO |  |
| 1.) | Does Store has separate water pumping connection? |  |  |
| 2.) | If Yes, U/G Water Tank Capacity of Building? |  |  |
| 3.) | If Yes, O/H Water Tank Capacity of Building? |  |  |
| 4.1.B | FIRE PUMP ROOM DETAILS:- | YES/NO |  |
| a.) | Pumping Arrangement: |  |  |
|  | Monoblock or Submersible? |  |  |
| b.) | Suction Type: |  |  |
|  | Positive or Negative? |  |  |
| c.) | Flow Type: |  |  |
|  | Riser or Downcomer? |  |  |
| d.) | Building Common Fire Pump Details: |  |  |
| 1.) | Electric Fire Pump – Main |  |  |
| 2.) | Electric Fire Pump – Standby |  |  |
| 3.) | Electric Fire Pump – Sprinkler |  |  |
| 4.) | Diesel Fire Engine |  |  |
| 5.) | Electric Fire Pump – Jockey |  |  |
| e.) | Booster Pump |  |  |
| 1.) | Does Store have separate fire pump connection? |  |  |
| 2.) | If Yes, Pump Type and Capacity? |  |  |

| 4.1.C | FIRE HYDRANT SYSTEM DETAILS:- | Details | Additional Remarks |
| --- | --- | --- | --- |
| a.) | Building Hydrant System Details: | YES/NO |  |
| 1.) | Internal Fire Hydrant System |  |  |
| i.) | Fire Hydrant Valve |  |  |
| ii.) | Fire Hose Box |  |  |
| iii.) | Fire Hose including accessories |  |  |
| iv.) | Hose Reel |  |  |
| v.) | Fire Extinguisher |  |  |
| vi.) | Fire Brigade Inlet |  |  |
| 2.) | Courtyard Fire Hydrant System | YES/NO |  |
| i.) | Fire Hydrant Valve |  |  |
| ii.) | Fire Hose Box |  |  |
| iii.) | Fire Hose including accessories |  |  |
| iv.) | Hose Reel |  |  |
| v.) | Fire Extinguisher |  |  |
| vi.) | Fire Brigade Inlet |  |  |
| b.) | Store Fire Hydrant System Details: |  |  |
| 1.) | Internal Fire Hydrant System | YES/NO |  |
| i.) | Internal Fire Hydrant Valve |  |  |
| ii.) | Fire Hose Box including Fire Hose and Accessories |  |  |
| iii.) | Fire Hose Reel |  |  |
| 2.) | External Fire Hydrant System | YES/NO |  |
| i.) | External Fire Hydrant Valve |  |  |
| ii.) | Fire Hose Box including Fire Hose and Accessories |  |  |
| iii.) | Fire Brigade Inlet |  |  |

| 4.1.D | FIRE SPRINKLER SYSTEM DETAILS:- | Details | Additional Remarks |
| --- | --- | --- | --- |
| 4.1.D.A | Does property has Fire Sprinkler Installed? | YES/NO |  |
|  | If Yes, Please furbish below information: |  |  |
|  | Sprinkler System is installed in basement. |  |  |
|  | Sprinkler System is installed in stilt parking. |  |  |
|  | Sprinklers are installed in common area of each floor |  |  |
|  | Sprinklers are installed inside complete premises |  |  |
| a.) | Building Sprinkler System Details: | YES/NO |  |
| 1.) | Internal Fire Sprinkler System |  |  |
| i.) | Pendent Sprinkler |  |  |
| ii.) | Upright Sprinkler |  |  |
| iii.) | Conceal Sprinkler |  |  |
| b.) | Basement Fire Sprinkler System | YES/NO |  |
| i.) | Pendent Sprinkler |  |  |
| ii.) | Upright Sprinkler |  |  |
| iii.) | Water Curtain |  |  |
| iv.) | Sidewall Sprinkler |  |  |
| 4.1.D.B | Does Store have separate Sprinkler Arrangement? | YES/NO |  |
| a.) | If Yes, Please mention the pressure in sprinkler line:- |  |  |
|  | If No, Is there Isolation Valve between the Property Sprinkler Line and Store Sprinkler line? |  |  |
|  | Has store installed Pressure Gauge in Sprinkler Line? |  |  |
|  | Has store installed Drain Valve in Sprinkler Line? |  |  |
| b.) | Store Sprinkler System Details: |  |  |
| i.) | Pendent Sprinkler |  |  |
| ii.) | Upright Sprinkler |  |  |
| iii.) | Conceal Sprinkler |  |  |

| 4.1.E | FIRE ALARM SYSTEM DETAILS:- | Details | Additional Remarks |
| --- | --- | --- | --- |
| 4.1.E.A | Does property has Fire Detection Installed? | YES/NO |  |
|  | If Yes, Please furbish below information: |  |  |
|  | Detection System is installed in basement. |  |  |
|  | Detection System is installed in parking. |  |  |
|  | Detectors are installed in common area of each floor |  |  |
|  | Detectors are installed inside complete premises |  |  |
| a.) | Building Fire Alarm System Details: | YES/NO |  |
| i.) | Fire Alarm Panel – Addressable |  |  |
| ii.) | Fire Alarm Panel – Conventional |  |  |
| iii.) | MCP |  |  |
| iv.) | Hooter |  |  |
| v.) | Smoke Detector |  |  |
| vi.) | Heat Detector |  |  |
| vii.) | Response Indicator |  |  |
| viii.) | Public Address System |  |  |
| ix.) | Talkback Units |  |  |
| 4.1.E.B | Does Store have separate Detection Arrangement? | YES/NO |  |
| i.) | Fire Alarm Panel – Addressable |  |  |
| ii.) | Fire Alarm Panel – Conventional |  |  |
| iii.) | MCP |  |  |
| iv.) | Hooter |  |  |
| v.) | Smoke Detector |  |  |
| vi.) | Heat Detector |  |  |
| vii.) | Response Indicator |  |  |
| viii.) | Public Address System |  |  |
| ix.) | Talkback Units |  |  |

| 4.1.F | PASSIVE FIRE FIGHTING SYSTEM DETAILS: | Details | Additional Remarks |
| --- | --- | --- | --- |
| 4.1.F.A | Building Details:- | YES/NO |  |
| a.) | Smoke Exhaust/ Ventilation System? |  |  |
| b.) | Staircase Pressurization System? |  |  |
| c.) | Auto Glow Fire Safety Signage? |  |  |
| d.) | LED Emergency Exit Signage? |  |  |
| e.) | Emergency Exit Doors? |  |  |
| f.) | Assembly Area? |  |  |
| g.) | Evacuation Plan Layout? |  |  |
| h.) | First Aid Kit? |  |  |
| i.) | Breathing Apparatus? |  |  |
| j.) | Fireman Axe? |  |  |
| h.) | Fire Bucket with Fire Sand? |  |  |
| i.) | Fire Beater? |  |  |
| j.) | Fire Blanket? |  |  |
| 4.1.F.B | Store Details:- | YES/NO |  |
| a.) | Smoke Exhaust/ Ventilation System? |  |  |
| b.) | Staircase Pressurization System? |  |  |
| c.) | Auto Glow Fire Safety Signage? |  |  |
| d.) | LED Emergency Exit Signage? |  |  |
| e.) | Emergency Exit Doors? |  |  |
| f.) | Assembly Area? |  |  |
| g.) | Evacuation Plan Layout? |  |  |
| h.) | First Aid Kit? |  |  |
| i.) | Breathing Apparatus? |  |  |
| j.) | Fireman Axe? |  |  |
| h.) | Fire Bucket with Fire Sand? |  |  |
| i.) | Fire Beater? |  |  |
| j.) | Fire Blanket? |  |  |

| 4.1.G | PORTABLE FIRE EXTINGUISHER: | Details | Additional Remarks |
| --- | --- | --- | --- |
| 4.1.G.A | Building Details:- | YES/NO |  |
| a.) | ABC TYPE FIRE EXTINGUISHER |  |  |
| b.) | CO2 TYPE FIRE EXTINGUISHER |  |  |
| c.) | DCP TYPE FIRE EXTINGUISHER |  |  |
| d.) | CLEAN AGENT FIRE EXTINGUISHER |  |  |
| e.) | K-TYPE FIRE EXTINGUISHER |  |  |
| f.) | MECHANICAL FOAM FIRE EXTINGUISHER |  |  |
| g.) | MODULAR FIRE EXTINGUISHER |  |  |
| h.) | AUTOMATIC GAS SUPPRESSION |  |  |
| i.) | FIRE BALL |  |  |
| 4.1.G.B | Store Details:- | YES/ NO |  |
| a.) | ABC TYPE FIRE EXTINGUISHER |  |  |
| b.) | CO2 TYPE FIRE EXTINGUISHER |  |  |
| c.) | DCP TYPE FIRE EXTINGUISHER |  |  |
| d.) | CLEAN AGENT FIRE EXTINGUISHER |  |  |
| e.) | K-TYPE FIRE EXTINGUISHER |  |  |
| f.) | MECHANICAL FOAM FIRE EXTINGUISHER |  |  |
| g.) | MODULAR FIRE EXTINGUISHER |  |  |
| h.) | AUTOMATIC GAS SUPPRESSION |  |  |
| i.) | FIRE BALL |  |  |

5.1.STORE INSPECTION SUMMARY

5.1.1. LIST OF AREAS INSPECTED IN STORE

a.) Common Assembly Point

b.) Storefront

c.) Entrance Lobby

d.) Retail Outlet Area

e.) Merchandise Display Area

f.) Sales Floor

g.) Trial Room

h.) Billing Section

i.) Stock Storage Area

j.) Scrap Area

k.) Pantry

l.) Electrical Room

m.) Administration room

n.) Employee Room

o.) Entry/Exit Point

p.) Emergency Exit Point

Apart above mentioned areas we also carried inspection and surveyed below mentioned critical areas:-

#### 1.) Back Areas

Stockroom / Inventory Storage, Goods Receiving Area, Employee Lockers / Changing Area, Staff Room

2.) Utility & Safety Infrastructure

Electrical Panel Room, HVAC / Air Handling Unit Area, Fire Control Room / Fire Panel Location

3.)Restroom Areas

Customer Restrooms, Staff Restrooms, Cleaning Supplies / Janitor Closet / Trash Disposal Area / Waste Storage

After carrying the inspection of above areas we derive HAZARD IDENTIFICATION report of any premises based on the type of material installed/ stored in the said location.

5.2.HAZARD IDENTIFICATION

| Sr No. | Area | Details of Fire Hazard Material/ Objects | Hazard Classification |
| --- | --- | --- | --- |
| 1.) | Store Entrance Lobby | Glass Facade, Automated Glass Doors, | Low-Hazard |
| 2.) | Retail Outlet Area | Clothes, Mannequins, Shelves, Spot Lights, LED Lights, Organized SS Trolleys | Low-Hazard |
| 3.) | Merchandise Display Floor | Mannequins, Shelves, Spot Lights, LED Lights, Wooden Compartmentation | Low-Hazard |
| 4.) | Cash Counter | Electrical wiring, POS systems, cash drawers, Computers, Electric devices | Mid-Hazard |
| 5.) | Main Exit Area | Glass Facade, Automated Glass Doors, | Low-Hazard |
| 6.) | Trial Rooms | No ignition source available. | Non-Hazardous |
| 7.) | Stock Storage Area | Clothes, Corrugated Boxes, Plastic Packaging, Cartons, Stocked Clothes | Hazardous |
| 8.) | Electrical / IT Panel Room | HV Electrical Panels, Cables, Sever Station, Cable Trays, | Hazardous |
| 9.) | Administrative Room | Cash Station, Computers, BMS/CCTV Room, Back office work desk | Mid-Hazard |
| 10.) | Pantry / Staff Kitchen | Coffee Vending Machine, Water Purifier, Cleaning Accessories | Non Hazardous |
| 11.) | Employee Rooms | No Ignition Source | Non-Hazardous |
| 12.) | Corridor/ Staircases | Low occupancy, no ignition source available. | Non-Hazardous |
| 13.) | Emergency Exit | No storage found, Clear means of Egress | Non-Hazardous |

5.3.OCCUPANCY LOAD DERIVATION

| Sr No. | Area | Prescriptive Occupant Load Factor (OLP) | Estimate Area in sq mtr. | Estimate People Assembly | Occupancy Load Derived |
| --- | --- | --- | --- | --- | --- |
| 1.) | Store Entrance Lobby | 2.6 m² per person |  | 5 |  |
| 2.) | Retail Outlet Area | 2.6 m² per person |  | 28 |  |
| 3.) | Merchandise Display Floor | 2.6 m² per person |  | 15 |  |
| 4.) | Cash Counter | 4.0 m² per person |  | 20 |  |
| 5.) | Main Exit Area | 2.6 m² per person |  | 10 |  |
| 6.) | Trial Rooms | 4.0 m² per person |  | 5 |  |
| 7.) | Stock Storage Area | 5.0 m² per person |  | 3 |  |
| 8.) | Electrical / IT Panel Room | 5.0 m² per person |  | 1 |  |
| 9.) | Administrative Room | 4.0 m² per person |  | 3 |  |
| 10.) | Pantry / Staff Kitchen | 4.0 m² per person |  | 2 |  |
| 11.) | Employee Rooms | 4.0 m² per person |  | 2 |  |
| 12.) | Corridor/ Staircases | 5.0 m² per person |  | 3 |  |
| 13.) | Emergency Exit | 5.0 m² per person |  | 3 |  |
|  |  |  |  | 100 |  |

(CALCULATION IS DERIVED BY ESTIMATING 100 PEOPLE ON AN AVERAGE IN STORE DURING PEAK HOURS)

5.4.FIRE LOAD CALCULATION

Fire load is the total heat energy (in megajoules, MJ) released per unit floor area (in square meters, m²)

Fire load is derived to obtain the loss severity of combustible materials in a space when burned.

Fire Load Formula:	Fire Load (MJ/m²)= ∑(mi×Hi) / A

Calorific value per kg = (MJ/kg) [MJ = MegaJoules]

Mi  =   Mass of combustible material (in kg)

Hi   =   Calorific value (heat of combustion) of each material (in MJ/kg)

A    =   Floor area of the store (in m²)

| Sr No. | Type of Material | Materials found in Store | Calorific Value (Per unit) | Mass of Material (Lumpsome) | Calorific Value (MegaJoule) |
| --- | --- | --- | --- | --- | --- |
| 1.) | Paper | Corrugated Box, Stationary, Loose Paper Items, Disposables, etc. | 15.4 |  |  |
| 2.) | Cotton | Clothes, Garments, Sweaters, Dress, Sweatshirt, Joggers, etc. | 15.8 |  |  |
| 3.) | Leather | Clothes, Blazers, Bags, Jackets, Shoes, Belts, Footwear, etc. | 17.6 |  |  |
| 4.) | Wood | Wooden Cabins, Partition Walls, Soft Board, Shelves, Cup Boards, etc. | 17.6 |  |  |
| 5.) | Wool | Coats, Blazers, Knitwear, Jumpers, Sweat Shirts & Pants, Sweaters, etc. | 19.6 |  |  |
| 6.) | Nylon | Clothes, Bags, Jackets, Shoes, Dress, Denims, Other Garments, etc. | 22.0 |  |  |
| 7.) | Polyester | Clothes, Garments, Dress, Tracksuits, Jumpers, Sweat Shirts & Pants, etc. | 22.0 |  |  |
| 8.) | Rubber | Footwear, Cables & Wires, Sportswear, Slippers & Sandals, Wearable, etc. | 37.4 |  |  |
| 9.) | Ethane | Ethanol, Profane and Butane used in making scent, deodorant, perfume,etc | 47.3 |  |  |
| 10.) | Polyethylene/ Polycarbonate | Plastic, PVC, XLPE, PE, Teflon, Wire & Cable Coatings, Other Electronics, etc | 48.4 |  |  |

TOTAL MEGA JOULES	(MJ)	=

TOTAL AREA OF STORE (A)	=

FIRE LOAD DENSITY(MJ/m2)	=	TOTAL MEGA JOULES	/ TOTAL AREA OF STORE

=	 	/

TOTAL FIRE LOAD DENSITY VALUE	= 	MJ/m2

Standard Fire Load Classification per NBC: IS 875 Part 2

| Fire Load Density (MJ/m²) | Classification |
| --- | --- |
| Up to 275 MJ/m² | Low Fire Load |
| 275–550 MJ/m² | Moderate Fire Load |
| Above 550 MJ/m² | High Fire Load |

REMARKS:

6.1.NBC NECESSITY

6.1. A) WATER TANK DETAILS

Height of Property	= 		Less than 15 Meters

Area of Property	=		____ m2

| Description | Storage Capacity Required (Per NBC) | Storage Capacity Provided on Site (At Actual) | Additional Remarks |
| --- | --- | --- | --- |
| Total U/G + O/H Domestic Water Tank |  |  |  |
| Total U/G + O/H Fire Water Tank |  |  |  |

ADDITONAL COMMENTS:

Above mentioned requirement is the minimum Fire Water Tank setup required for properties of

Less than 15m height and area of > 500 m² as per NBC.

REMARKS:

6.1. B) FIRE PUMP DETAILS

Height of Property	= 		Less than 15 Meters

Area of Property	=		____  m2

| Description | Requirement (Per NBC) | Availability (At Actual) | Additional Remarks |
| --- | --- | --- | --- |
| FIRE PUMP/ SPRINKLER PUMP -MAIN |  |  |  |
| FIRE PUMP/ SPRINKLER PUMP -STANDBY/ DIESEL |  |  |  |
| FIRE PUMP -JOCKEY |  |  |  |
| FIRE PUMP -BOOSTER |  |  |  |

ADDITONAL COMMENTS:

Above mentioned requirement is the minimum Fire Pump setup required for properties of

Less than 15m height and area of > 500 m² as per NBC.

REMARKS:

6.1. C) FIRE HYDRANT DETAILS

Height of Property	= 		Less than 15 Meters

Area of Property	=

| Description | Requirement (Per NBC) | Availability (At Actual) | Additional Remarks |
| --- | --- | --- | --- |
| FIRE HYDRANT VALVE – YARD | ACCESSIBLE AT EVERY 30 METERS |  |  |
| FIRE HYDRANT VALVE – INTERNAL | EACH FLOOR NEAR STAIRCASE |  |  |
| FIRE HOSE BOX/ CABINET | EACH FLOOR NEAR STAIRCASE |  |  |
| CANVAS FIRE HOSE | EACH FLOOR NEAR STAIRCASE |  |  |
| FIRE HOSE REEL DRUM | EACH FLOOR NEAR STAIRCASE |  |  |
| FIRE BRIGADE INLET/OUTLET | EASILY ACCESIBLE TO FIRE TENDER |  |  |

ADDITONAL COMMENTS:

Above mentioned requirement is the minimum Fire Hydrant setup required for properties of

Less than 15m height and area of > 500 m² as per NBC.

REMARKS:

6.1. D) FIRE SPRINKLER/ SUPPRESSION SYSTEM DETAILS

Area of Store		=		_____ m2

| Description | Requirement (Per NBC) | Availability (At Actual) | Additional Remarks |
| --- | --- | --- | --- |
| SPRINKLER BULB- PENDANT | Required every 3 X 3 Meter of Area, Must cover minimum 9 mtr2 Area from one point to other |  |  |
| SPRINKLER BULB- SUPPRESSION | Must Install in Server Area, Kitchen |  |  |

ADDITONAL COMMENTS:

Per NBC, Sprinkler bulb must not exceed 3 meters of area vertically or horizontally and shall be installed at every 3 mtr2 area having water flow rate of minimum 15–20 L/min/m² each covering area of 5 mtr2 once discharged and must maintain 7 bar pressure at closest point with 3.5–4 bar at remote point.

We inspected the store and found:-

Isolation Valve (Butterfly valve) -

Pressure Gauge -

Drain Line -

REMARKS:

6.1. E) FIRE ALARM SYSTEM/ PUBLIC ADDRESS SYSTEM DETAILS

Area of Store		=		_____ m2

| Description | Requirement (Per NBC) | Availability (At Actual) | Additional Remarks |
| --- | --- | --- | --- |
| FIRE ALARM PANEL | Addressable or Conventional |  |  |
| PUBLIC ADDRESS SYSTEM | Recommended but not Compulsory |  |  |
| MANUAL CALL POINT | Near Staircase, Entry and Exit Point |  |  |
| HOOTER CUM FLASHER | Minimum 65 Decibel range at each floor |  |  |
| SMOKE DETECTOR | Required covering 9 mtr2 area per device |  |  |
| HEAT DETECTOR | Required covering 9 mtr2 area in electrical room |  |  |
| BATTERY BACKUP | Upto 24 Hours |  |  |

ADDITONAL COMMENTS:

Above mentioned requirement is the minimum Fire Detection System required for mercantile properties irrespective of size and area of the store as per NBC for Mercantile Occupancy and must undergo timely maintenace under IS 2189:2019

REMARKS:

6.1. F) FIRE EXTINGUISHER SYSTEM

Area of Store		=		_____ m2

| Description | Requirement (Per NBC) | Availability (At Actual) | Additional Remarks |
| --- | --- | --- | --- |
| CO2 TYPE FIRE EXTINGUISHER | Each required covering 200 mtr2 area (4.5 KG) |  |  |
| ABC TYPE FIRE EXTINGUISHER | Each required covering 30 mtr2 area (6.0 KG) |  |  |
| MODULAR FIRE EXTINGUISHER | Each required in storage & electrical area |  |  |
| CLEAN AGENT EXTINGUISHER | Each required near Electrical Room |  |  |

ADDITONAL COMMENTS:

Above mentioned requirement is the minimum Fire Extinguisher System required for mercantile properties irrespective of size and area of the store as per NBC for Mercantile Occupancy and must undergo timely maintenace under IS 2190:2010

REMARKS:

6.1. G) PASSIVE FIRE FIGHTING SYSTEM

| Description | Requirement (Per NBC) | Availability (At Actual) | Additional Remarks |
| --- | --- | --- | --- |
| FIRE BUCKET | 10 Litre Capacity Bucket required per 100 mtr2 | NOT AVAILABLE | Needs to install |
| FIRE EXIT DOOR | Each required at travel distance of 100 mtr2 | AVAILABLE | Meets requirement |
| FIRE EXIT PASSAGE | Min passage of 1Mtr Breadth X 2Mtr Height Required for 50 People | AVAILABLE | Meets requirement |
| LED EXIT SIGNAGES | Mounted 3.0mtr above, Visible upto 30 Mtrs | AVAILABLE | Meets requirement |
| FIRE SAFETY SIGNAGES | Stick/ Mounted 2.5 mtr above, visible at 15mtrs | AVAILABLE | Meets requirement |
| AUTOMATED ENTRY/ EXIT DOOR | Must open automatically after power failure | AVAILABLE | Meets requirement |
| SMOKE EXHAUST SYSTEM | Required At Basement Only | NOT AVAILABLE | Store is Situated at Ground Floor |
| REFUGE/ ASSEMBLY AREA | Not mandatory/ 0.5 m2 per person | AVAILABLE | Meets requirement |

REMARKS:

7.1 EMERGENCY EVACUATION CHECKLIST

| SR. | CHECKLIST ITEM | YES/NO/N.A. | REMARKS |
| --- | --- | --- | --- |
| 1 | Are all fire exits clearly marked with illuminated “EXIT” signs? | YES | Exits are marked clearly |
| 2 | Are Exit signs always visible and not obstructed inside the store? | YES | Exit Signs are unobstructed |
| 3 | Are fire exits unlocked and accessible during business hours? | YES | Fire Exits are accessible |
| 4 | Are exit doors easy to open (no special keys/tools required)? | YES | Easy to Open |
| 5 | Do all fire exits open in the direction of escape (outwards)? | YES | Yes, they open outwards |
| 6 | Is the number of exits adequate for the occupancy load? | YES | Yes, meets NBC Requirements |
| 7 | Are fire exit routes free from obstructions (boxes, clothing racks, etc.)? | YES | No obstacles found |
| 8 | Are corridors and staircases leading to exits kept clear at all times? | YES | Staircases available |
| 9 | Are exit routes of appropriate width (minimum width per code)? | YES | Minimum width provision Maintained |
| 10 | Are exit paths well-lit, including emergency lighting? | YES | Yes, emergency lights available |
| 11 | Are fire exits free from flammable material storage nearby? | YES | Yes, no flammable materials found near fire exit |
| 12 | Are exit doors not blocked by security grills or roller shutters (or have quick-release)? | YES | No grills or shutters found near fire exit door |
| 13 | Are floor-level directional signs present along the escape path? | YES | Directional signs are available |
| 14 | Is there a fire escape plan posted and visible to staff and public? | YES | Fire Escape plan is available however needs to be bigger |
| 15 | Are employees trained on emergency evacuation routes and assembly points? | YES | Yes, employees are trained. |

7.2 SMOKE VENTILATION CHECKLIST

| No. | Checklist Item | Yes / No / N.A. | Remarks |
| --- | --- | --- | --- |
| 1 | Is a smoke ventilation system installed (natural or mechanical)? | YES | NATURAL |
| 2 | Are automatic smoke vents present in high-risk areas (e.g. stockrooms, storage, upper floors)? | NO | No automatic smoke vents available in store |
| 3 | Are manual smoke ventilation controls easily accessible and clearly marked? | NO | Manual smoke ventilation is complex to access |
| 4 | Is the operation of smoke ventilation systems linked to the fire alarm or detection system? | NO | Smoke vents are not automated |
| 5 | Are natural vents (e.g., windows, roof hatches) unobstructed and operational? |  |  |
| 6 | Are mechanical smoke extraction fans functional and tested regularly? | NA | Mechanical smoke extraction fans not installed |
| 7 | Are there any designated smoke shafts or smoke lobbies in buildings? | NA | Building is at ground Level only |
| 8 | Are fire/smoke dampers installed in HVAC systems to prevent smoke spread? | NO | No HVAC Installed in property |
| 9 | Are maintenance records for the smoke ventilation system available and up to date? | NA | Not required |
| 10 | Are staircases and escape routes protected with smoke control systems (e.g., pressurization)? | NO | No smoke control systems installed |
| 11 | Is there signage indicating location and function of smoke vents or controls? | NA | Automated Smoke ventilation not required |
| 12 | Are smoke detectors in place to trigger smoke ventilation systems in relevant zones? | NO | Automated Smoke ventilation not installed |
| 13 | Are emergency power supplies (UPS or generator) available for mechanical ventilation systems? | NO | Automated Smoke ventilation not installed/ not required |
| 14 | Are tenants or occupants trained on the function and use of smoke ventilation features? | NO | No smoke ventilation system installed in premises |

7.3 ELECTRICAL SAFETY CHECKLIST

| No. | Checklist Item | Yes / No / N.A. | Remarks |
| --- | --- | --- | --- |
| 1 | Are all the electrical installations, conducted, safe and comply IS Standards? | YES | Electrical Installation are safe |
| 2 | Any signs of overheating of any socket, outlet point, device or appliances? | NO | No signs detected |
| 3 | Are all switches and sockets in good condition and connected to MCB Junction? | YES | All switches and sockets are working under ELV |
| 4 | Are any Cables or Cords frayed, damaged, tangled or joined with ductile tape? | NO | No cables are tangled/ damaged |
| 5 | Are all the electrical panels accessible, covered labeled and placed safely? | YES | All electrical panels ARE accessible, covered & labeled. |
| 6 | Are extension cords used in Store to connect High Output Electrical Appliances? | NO | No extension cords are used in Store. |
| 7 | Are all the electric devices in store turned off when not in use? | YES | All devices in store are turned off after closing of store. |
| 8 | Do all electric equipments have proper ventilation and clearance? | NO | Ventilation has scope of improvement. |
| 9 | Does Store have Maintenance logs for all electrical systems? | YES | Store maintains documentation and logs. |
| 10 | Is Emergency shutoff for high-power equipment marked and accessible? | YES | Emergency shut off shut on are accessible. |
| 11 | Are any combustible materials stored near electrical sources? | NO | No combustible materials found near electrical sources. |
| 12 | Have Stores placed CO2 Type Fire Extinguishers near Medium/High Hazard Electrical Areas? | YES | Yes, CO2 fire extinguishers are placed near electrical room. |
| 13 | Does any electrical equipment emit unusual heat, sparks or noise? | NO | No equipment emit spark, noise or heat. |

7.4 MAINTENANCE/TRAINING CHECKLIST

7.4.1. FIREFIGHTING SYSTEM MAINTENANCE

| Item | Maintenace Status (Maintained/Not Maintain) | Maintenace Frequency (Monthly/Quarterly/Annually) | Report Status (Available/ Not Available) | REMARKS |
| --- | --- | --- | --- | --- |
| FIRE PUMPS |  |  |  |  |
| FIRE HYDRANT SYSTEM |  |  |  |  |
| FIRE SPRINKLER SYSTEM |  |  |  |  |
| FIRE ALARM SYSTEM |  |  |  |  |
| FIRE EXTINGUISHERS |  |  |  |  |
| FIRE SUPPRESSION SYSTEM |  |  |  |  |
| FIRE EXIT DOORS |  |  |  |  |

7.4.2. FIRE SAFETY TRAINING

| Description | Remark | Comments |
| --- | --- | --- |
| Does staff received fire safety induction training? |  |  |
| Are Fire Evacuation Drills Conducted? |  |  |
| Does Store have QRT/ Fire Wardens/ Fire Volunteers? |  |  |
| Does all staff know Extinguisher Locations & usage basics? |  |  |
| Does all staff know Fire Evacuation Points and Exits? |  |  |

7.4.3. DOCUMENTATION

| Description | Remark | Comments |
| --- | --- | --- |
| Fire Equipment Service Records? |  |  |
| Fire Mock Drill Records? |  |  |
| Staff Fire Safety Training Records? |  |  |
| Fire System Testing Records? |  |  |
| Security Team Fire Safety Training Records? |  |  |
| AMC and Repair Reports of Fire System? |  |  |

ADDITIONAL COMMENTS:

Please maintain testing reports, AMC reports of firefighting equipments carried inside the store and we recommend the store to carry firefighting testing of fire equipments of store from 3rd party contractor on regular interval to have the count of working and not working firefighting equipments.

We request store authority to maintain documentation of trainings and mock drills carried for the employees, support staff, security of the store as when requested we were not provided any documents of undergone first aid fire safety training of employees working in store.

8.1. RISK IDENTIFICATION

8.2 OBSERVATION AND REMARKS

8.3 AREAS OF IMPROVEMENT

9.1 CONCLUSION

The retail outlet shows a satisfactory level of fire safety preparedness. With the implementation of the above recommendations, the fire risk level can be further reduced and compliance improved. The fire safety audit was conducted in accordance with applicable standards and statutory requirements. The audit assessed the adequacy and compliance of fire safety measures including:

- Fire detection and alarm systems

- Emergency exits and evacuation plans

- Fire-fighting equipment

- Electrical fire risk controls

- Training and awareness programs

- Maintenance and documentation practices

We sincerely thank you for the opportunity to conduct the fire safety audit at your esteemed premises.

Your proactive approach toward ensuring a safe and compliant environment is commendable. Cooperation from your team during the audit process greatly contributed to the successful and thorough assessment. We hope this report assists in enhancing fire safety measures and promoting a culture of safety within your organization. Should you require any further guidance or assistance in implementing the recommendations, please feel free to reach out.

We look forward to continuing our professional relationship and supporting your ongoing safety initiatives.

Regards,

FIREFIGHTING EQUIPMENTS STORE WISE LOCATION FOR REFERENCE

| A | FIRE HYDRANT VALVES | Maintenance Status | Additional Remarks |
| --- | --- | --- | --- |
| B | FIRE HOSE WITH ACCESSORIES | Maintenance Status | Additional Remarks |
| C | FIRE HOSE REEL | Maintenance Status | Additional Remarks |
| D | FIRE ALARM PANEL | Maintenance Status | Additional Remarks |
| E | MANUAL CALL POINT | Maintenance Status | Additional Remarks |
| F | HOOTER | Maintenance Status | Additional Remarks |
| G | SMOKE DETECTOR | Maintenance Status | Additional Remarks |
| H | FIRE EXTINGUISHER | Maintenance Status | Additional Remarks |